Project 9: Stepper Motor Precision Positioner

Abstract

This project implements a precision stepper motor control system using an Arduino Uno, 28BYJ-48 stepper motor, ULN2003 driver, potentiometer, push buttons, and OLED display. The system allows accurate motor positioning by converting potentiometer input into target angles and controlling motor movement using clockwise, counter-clockwise, and home position buttons. Real-time angle, target position, step count, and motor direction are displayed on an OLED screen.

Problem Statement

Modern CNC machines, robotic arms, and 3D printers require precise motor positioning. The objective of this project is to design a stepper motor controller capable of accurately moving to desired angular positions while continuously tracking motor position and displaying system information.

Objectives

- Control a 28BYJ-48 stepper motor with precise step positioning.

- Read potentiometer input and convert it into a target angle between 0° and 360°.

- Rotate the motor clockwise and counter-clockwise using push buttons.

- Implement a home position reset function.

- Display current angle, target angle, direction, and step count on OLED.

- Track motor position using software.

Components Used

| Component | Quantity |
| --- | --- |
| Arduino Uno R3 | 1 |
| 28BYJ-48 Stepper Motor | 1 |
| ULN2003 Driver Board | 1 |
| OLED Display SSD1306 | 1 |
| 10kΩ Potentiometer | 1 |
| Push Buttons | 3 |
| LED | 1 |
| Breadboard | 1 |
| Jumper Wires | As Required |

Circuit Connections

ULN2003 Driver

| ULN2003 Pin | Arduino Pin |
| --- | --- |
| IN1 | D8 |
| IN2 | D9 |
| IN3 | D10 |
| IN4 | D11 |
| VCC | 5V |
| GND | GND |

Potentiometer

| Potentiometer Pin | Arduino |
| --- | --- |
| Left Pin | GND |
| Right Pin | 5V |
| Middle Pin | A0 |

Push Buttons

| Button | Arduino Pin |
| --- | --- |
| CW Button | D2 |
| CCW Button | D3 |
| Home Button | D4 |

LED

| LED | Arduino Pin |
| --- | --- |
| Indicator LED | D7 |

OLED Display

| OLED Pin | Arduino Pin |
| --- | --- |
| SDA | A4 |
| SCL | A5 |
| VCC | 5V |
| GND | GND |

Working Principle

The potentiometer is used to set the desired target angle. The Arduino continuously reads the potentiometer value and maps it between 0° and 360°.

The clockwise button rotates the motor by a fixed number of steps in the clockwise direction. The counter-clockwise button rotates the motor in the opposite direction. The home button resets the current position to zero without physically moving the motor.

The OLED display provides real-time information about:

- Current Angle

- Target Angle

- Direction

- Total Steps

Algorithm

- Initialize OLED display and stepper motor.

- Read potentiometer value.

- Convert potentiometer value into target angle.

- Monitor button inputs.

- If CW button is pressed:

  - Rotate motor clockwise.

  - Update step count.

- If CCW button is pressed:

  - Rotate motor counter-clockwise.

  - Update step count.

- If Home button is pressed:

  - Reset current position to zero.

- Calculate current angle.

- Update OLED display.

- Print data to Serial Monitor.

Software Logic

Stepper Motor Configuration

const int STEPS_PER_REV = 2048;

Stepper motor(STEPS_PER_REV, 8, 10, 9, 11);

Motor Speed

motor.setSpeed(10);

Angle Calculation

angle = (currentSteps % 2048) * 360.0 / 2048.0;

CW Rotation

motor.step(256);

CCW Rotation

motor.step(-256);

Home Position

currentSteps = 0;

OLED Display Output

Example:

Current : 315°

Target  : 350°

Steps   : -256

Dir     : CCW

Serial Monitor Output

Example:

Current Angle : 315.00

Target : 350

Steps : -256

Direction : CCW

Applications

- CNC Machines

- 3D Printers

- Robotic Arms

- Position Control Systems

- Industrial Automation

- Camera Positioning Systems

Advantages

- High positioning accuracy

- Low cost implementation

- Easy to control

- Reliable operation

- Real-time monitoring

Future Scope

- Mobile App Control

- Bluetooth Monitoring

- Wi-Fi Based Control

- Automatic Position Tracking

- Industrial Automation Integration

Conclusion

The Stepper Motor Precision Positioner successfully demonstrates accurate angular positioning using a 28BYJ-48 stepper motor. The system uses a potentiometer for target position selection, push buttons for directional control, and an OLED display for real-time feedback. This project provides a strong foundation for understanding the motion control principles used in CNC machines, robotic systems, and industrial automation.

Prepared By:
Puthiyaraj P
IIT Jammu Summer School 2026 – Home Assignment Project 9